[Link to QEs 29-52] [Link to QEs 80]

QE #53 (0.00 hrs) 21Feb19 [D] Topic: New battery order form (after re-sizing from ME testing). Outcome(s): Have new order for batteries.

QE #54 (0.50 hrs) 21Feb19 Topic: Component placement. - On Device: - Arduino - Buttons - Display - SD breakout - Load cell amplifier - Motor controller - On Battery Pack: - Batteries - Fuses - Power button On Device. Item; Height; Width; Depth; Weight. Arduino. Buttons (each). Display. SD breakout. Amplifier. Motor controller.; On Battery Pack. Item; Height; Width; Depth; Weight. Batteries (each). Array. Fuses (each). Array. Power button.. Outcome(s): Have the competents list to tell MEs where each component should go.

QE #55 (1.00 hrs) 21Feb19 [GP&A] (Whole team) Topic: Midterm testing presentation peer reviews. Outcome(s): Have documentation for watching, analyzing, and critiquing other teams’ presentations.

QE #56 (1.50 hrs) 21Feb19 [A] Topic: Reading the forum that Ian Alexander found that has (what he called) a motor controller code (with PID) that can do everything we need. - (Arduino Forum) Topic: DC motor control with PID [from “kas”] - kas’s project was with a balancing robot using 2 350 RPM Pololu motors with integrated encoders (with a 14A dual motor driver). From August 2010. - Codes (2 parts): - First Code - motorForward()…functions that are defined in main - motorBackward() - motorStop() - Pass a PWM value [0, 255] - PID has control over the motor speeds - It’s a lot, but maybe it could be made into a library or just piecemeal-ed into the code. - There are a lot of functions, but they look workable and customizable. - Conclusion/Prescription - I think that this code seems very likely to provide what this project needs and could be cleaned and serviced. - Ian should figure out the pinout to suit our project and should make all appropriate changes in the code. - The code comments need to be check for clarity, but seem good quality as is. - Proper citation needs to be created and inserted. - Citation: Outcome(s): This code will get the motor control (with PID) that is required.

QE #57 (2.00 hrs) 23Feb19 [M] Topic: Outlining code and fleshing out individual functions. Outcome(s): The code is close to being ready for initial testing. Need to add: error handling and motor/PID control.

QE #58 (1.00 hrs) 24Feb19 [A] Topic: Analyzing the code Ian found for compatibility with the present code and for how to integrate. I had to go back and look at my control notes before I mis-identified the error. Outcome(s): Switching back to the PID library (etc.) I found earlier.

QE #59 (1.00 hrs) Feb19[A] Topic: Analyzing the PID code I found for compatibility with the present code and for how to integrate. Outcome(s): I will be attempting to use the proportional on measure PID type control in order to help prevent overshoot.

QE #60 (1.00 hrs) 26Feb19 [GP&A] (Emily, Dylan, and Me) Topic: Team meeting (just the MEs and myself) and working on next report due. - MEs doing analysis on the newest frame design. - MEs need to put their tests into the required document form. - Need foam sample from NASA, but they are ignoring us. - Encoder: - Moved to motor… - Because the LA casing can’t handle the load weight; therefore, the previous encoder placement plan wouldn’t work. - The gear ratios, etc., are unknown. - The new casing design might require to be cut to specs if can’t bend the metal. - Dylan will count the gears for the ratio - I might need to change the buttons. - Making sure items to go in the device are going to fit and how the design needs to be changed. - Now we have to make our own attachment piece… - Dylan asked if the load cell would “make the load go up even though the sample piece isn’t getting pulled.” I explained how a load cell works, and Emily Outcome(s): Got the teammates that showed up caught up and discussed issues with device to be solved.

QE #61 (1.50 hrs) 28Feb19 [GP&A] (Whole team) Topic: Team meeting with Dr. Guo - Encoder - DrG asked TC if he’d worked on it…TC: need to test if it works - EZ: told about how shop gut suggested to move the encoder in order to support case - Talking about the case to hold the motor…how to create it - DrG concerned about how manufacturing will affect the structural integrity - IA: asking how COG will be affected…EZ: haven’t chosen material, but shouldn’t be an issue - DrG expressed some concern about the total length of the device (~15”) - DrG: do we need all 2” of stroke - Buttons: - EZ: explained function…DrG: can it just stay once done? YEP - Talked about how ours will be compared. It’s required…DrG: don’t worry about that…just talk about advantages. - DrG: don’t compare the devices…WTF they said we need to compares them - DrG: can’t say new tests are better than old test…you tell them we can meet the specs.. - EZ: We (DB and/or TC) will go talk to the shop guy to see how/if what is designed can be done - EZ is going to work on resizing the battery case and getting models fixed up. - We need to re-measure all the components in order to get accurate measurements Outcome(s): Have tasks for week(s) ahead and cleared up some component functionality.

QE #62 (1.50 hrs) 28Feb19 [A] Topic: Outlining the library Thommy wants to use for the motor driver. - Arduino Library - Function Definitions - Setup() - Baud is 115200…need to check if it will work at 9600 - “void setM1Speed(int speed)” - Sets Motor 1 so a speed where speed is [-400, 400] - 400 corresponds to motor current flowing from M1A to M1B. - -400 corresponds to motor current flowing from M1B to M1A. - “unsigned getFault()” - Returns “1” if there is a motor driver fault, “0” if no fault. - “DualMC33926MotorShield(…)” - Allows for custom mapping of pins - “void init()” - “Initialize pinModes and maybe timer1.” - User’s Guide - Dual motor driver - FET for battery protection - Stackable on the Arduino - Logic inputs are 5 ¦ 3.3 V Arduino Pin; Shield Pin Name; Basic Function. Digital 4; D2 (or nD2); Tri-state disables both outputs of both motor channels when LOW; toggling resets latched driver fault condition. Digital 7; M1DIR; Motor 1 direction input. Digital 8; M2DIR; Motor 2 direction input. Digital 9; M1PWM; Motor 1 speed input. Digital 10; M2PWM; Motor 2 speed input. Digital 12; SF (or nSF); Status flag indicator (LOW indicates fault). Analog 0; M1FB; Motor 1 current sense output (approx. 525 mV/A). Analog 1; M2FB; Motor 2 current sense output (approx. 525 mV/A). - Between 5-8V degrades motor controller performance and max current output - Max continuous current rating of 5A - Will reduce current automatically if over or if temp reaches limit - They tested at room temp (no forced air + 100 =d): max, lasted 10 s; 4A, 37 s; 3A, 10 min. - They suggest to add cap(s) in order to reduce noise. - Simplified Motor Control Truth Table Inputs; Outputs. D2; MxPWM; MxDIR; MxA; MxB; SF; Mode. H; H; L; H; L; H; forward. H; H; H; L; H; H; reverse. H; L; X; L; L; H; brake. L; X; X; Z; Z; L; coast. L = LOW, H = HIGH, X = HIGH or LOW, Z = high impedance - Customizing the Shield - The yellow boxes would need their trace cut if any custom mapping is to occur. Outcome(s): Have clear idea of how to work the library and how the board operates.

QE #63 (2.00 hrs) 28Feb19 [M] Topic: Adding code for motor controller and integrating PID controls. - I did my best to setup the PID, but I need Thommy to confirm how the encoder will output to determine how the input to the encoder will work. - Need to go throw and count the added delay() functions because otherwise the code will take too long.. Outcome(s): Have the PID integrated (or started to) and I now have a method of capturing an input for the PID from the encoder.

QE #64 (1.00 hrs) 28Feb19 [A] Topic: Calculating the gear ration and figuring the rotational motion of motor to equivalent linear motion of the hook assembly. Gears separated; Gears assembled in housing. - Counting the number of teeth on each gear yielded: - A: 12 - B: 12 and 30 - C: 11 and 36 - D: 45 - Total Gear Ratio with compound gears: - Gear Ratio=Driven Gear TeethDrive Gear Teeth - 301236124511=30*36*4512*12*11=30.681 - Conversion to linear motion - 1 rotation of lead screw is 1/16th of inch linear - 1 rotation of motor=130.681= 0.3259 rotations of lead screw - 1 rotation of motor=130.681116= 0.00204 inches of linear motion - # rotations of motor to get 116 inch of linear motion=30.681 motor rotations - # rotations of motor to get 1 inch of linear motion=30.68116=490.0 motor rotations - # rotations of motor to get 2 inches of linear motion=30.68132=981.81 motor rotations - # rotations of motor to get 3 inches of linear motion=30.68148=1472.72 motor rotations - For code: - Inputs linear set point in inches - Outputs rotational speed in rotations - Need to check if the speed of motor is 2356.55 rotationsminute (3 inchesminute) - Using a delay(): 1472.73rotmin=1472.73 rot1 minx1 min60 sx1 s103 ms=0.0245455 rotms ∴ 100 ms≅2.45455 rotations⇒2 rotations Outcome(s): Have the gear ratio and a conversion from rotational motion of the motor to linear motion of the hook assembly.

QE #65 (0.50 hrs) 28Feb19 [T] Topic: Creating test plans and documentation for the battery charger test. Outcome(s): Have required documentation format for the battery charger test.

QE #66 (7.50 hrs) 28Feb19 [T] Topic: Testing the battery charger for functionality and timing. Outcome(s): Battery charger works, but it does take longer than I expected/hoped.

QE #67 (0.00 hrs) 06Mar19 Topic: BTB #3. Outcome(s): Have documentation for BTB #3.